Circuit Analysis LAB 6 Explanation

VERIFICATION OF KIRCHHOFF’S LAWS (KCL & KVL)

Objective

To verify Kirchhoff’s Current Law (KCL) and Kirchhoff’s Voltage Law (KVL) using practical circuits and measurements.

Apparatus Required

Regulated Power Supply, Digital Multimeter, Breadboard, Connecting Wires, Resistors (330Ω, 220Ω, 1kΩ).

Theory

Kirchhoff’s Current Law (KCL)

KCL states that the total current entering a node is equal to the total current leaving the node. This is based on the conservation of charge.

Kirchhoff’s Voltage Law (KVL)

KVL states that the algebraic sum of all voltages in a closed loop is zero. This is based on the conservation of energy.

Procedure for KCL

1. Connect the circuit as per diagram.
2. Set voltage using power supply.
3. Measure currents I1, I2, and I3.
4. Repeat for different voltages.
5. Verify I1 = I2 + I3.

Observations for KCL

At 5V: I1 = 5.69mA, I2 = 3.12mA, I3 = 2.56mA.
At 15V: I1 = 17.06mA, I2 = 9.37mA, I3 = 7.68mA.
At 25V: I1 = 28.43mA, I2 = 15.62mA, I3 = 12.81mA.
Thus, I1 ≈ I2 + I3.

Procedure for KVL

1. Connect the circuit as per diagram.
2. Apply voltage using RPS.
3. Measure voltages V1, V2, and V3.
4. Repeat for different values.
5. Verify E1 = V1 + V2.

Observations for KVL

At 5V: V1 = 3.23V, V2 = 1.77V.
At 10V: V1 = 6.46V, V2 = 3.54V.
At 15V: V1 = 9.69V, V2 = 5.31V.
Thus, E1 = V1 + V2 is verified.

Result

Both Kirchhoff’s Current Law and Kirchhoff’s Voltage Law are successfully verified through this experiment.