Circuit Analysis Lab Report
Operational Amplifiers (Lab 7)

Objective

To investigate the input-output relationship of inverting and non-inverting amplifiers using IC 741 operational amplifier.

Theory

Operational amplifiers are high-gain voltage amplifiers used in various configurations. In this experiment, two configurations are studied:

1. Inverting Amplifier: Produces output 180° out of phase with input.
2. Non-Inverting Amplifier: Produces output in phase with input.

The IC 741 op-amp consists of 8 pins where pin 2 is inverting input, pin 3 is non-inverting input, pin 6 is output, and pins 4 and 7 are supply terminals.

Inverting Amplifier Explanation

In the given circuit, the input signal is applied to the inverting terminal through resistor R1 (1kΩ), while feedback resistor R2 (2kΩ) connects output to input. The non-inverting terminal is grounded.

The gain is calculated as:
Av = -Rf/Rin = -2k/1k = -2

This means the output is inverted and amplified twice the input. The oscilloscope shows a sine wave input and an inverted output waveform.

Non-Inverting Amplifier Explanation

In this configuration, the input signal is applied to the non-inverting terminal. The inverting terminal is connected to a feedback network of R1 (1kΩ) and R2 (3kΩ).

The gain is calculated as:
Av = 1 + (Rf/R1) = 1 + (3k/1k) = 4

The output is amplified four times and remains in phase with the input signal.

Apparatus

• IC 741 Operational Amplifier
• Resistors (1kΩ, 2kΩ, 3kΩ)
• Signal Generator
• Oscilloscope
• Power Supply
• Breadboard and wires

Procedure Summary

For both circuits, connections were made as per diagrams. A sinusoidal input was applied and both input and output waveforms were observed using an oscilloscope.

Results & Observations

Inverting amplifier produced an inverted waveform with gain ≈ -2.
Non-inverting amplifier produced an in-phase amplified waveform with gain ≈ 4.
The practical results matched theoretical expectations.

Conclusion

The experiment successfully demonstrated the behavior of inverting and non-inverting amplifiers. The gain and phase shift characteristics were verified using IC 741.